This project demonstrates how to prevent Cloudflare-style outages caused by oversized, malformed, or dangerous configuration updates—implemented on the STM32 Nucleo-F103RB.

The firmware receives configuration packets over UART, validates size + bounds, and applies them safely. It includes:

- Hardware-level fault recovery using the Independent Watchdog (IWDG)

- Safe-boot mode with a 8-second UART "escape window"

- Atomic config load + validation

- Freeze-fault testing (simulates a hung control-plane)

- UART logging for diagnostics

- LED heartbeat showing live configuration (blink rate changes based on config)

The demo replicates the concept of how misconfigurations could crash Cloudflare’s fleet — and shows how to prevent such cascaded failures on embedded systems.

Hardware Requirements

| Item | Notes |
| --- | --- |
| STM32 Nucleo-F103RB | Main MCU board |
| USB cable | For flashing & UART |
| Onboard LED (LD2) | Used as system heartbeat |
| Optional: RealTerm / PuTTY | For UART testing |
| Optional: Python 3.x | For automated test scripts |

Firmware Features

✔ Safe Boot (Startup Protection)

At power-on, the device waits 8 seconds for any UART key press.
If pressed → enters Safe Mode (IWDG disabled).
If not → Watchdog enabled (3 seconds timeout).

Real term Ouput